|  | Inbouw-chutemagneet - 558 mm Artikelnummer: E0153546 |
| --- | --- |

Kenmerken

| Omschrijving | Inbouw-chutemagneet - 558 mm |
| --- | --- |
| Artikelnummer | E0153546 |
| Product key | SCH-M-N-204x55-0400-B-B-B |
| Min. Fe-particle size that can be caught | 0,5 mm |
| Production stop required during ferrous particles disposal | Yes |
| Cleaning (ferrous particles disposal) method | Manual |
| Ferrous particles disposal via | Manually by moving away the extractor plate |
| Ferrous particles outlet size | 420x224 mm |
| Interface connection type/size | Mounting hole Ø11 mm (4x) |
| Materials in contact with product | AISI316L (SS 1.4404) |
| Material seal between housing and door/magnet unit | Silicone VMQ, 20° Shore, EC1935/FDA, white |
| Surface finishing (interior/exterior) | S1: Grit blasted, Ra 3 µm |
| Welds inside the product channel | WE: Continuously welded |
| Welds outside the product channel | WE: Intermittently welded |
| Max. operating temperature | 60 °C |
| Min./max. ambient temperature | -20 to 60 °C |
| Magnetic system | Permanent Neoflux™ magnet |
| Magnet quality | Neodymium N-35 |
| Magnet dimensions | 400x204x55 mm |
| Number of magnets | 1 |
| Magnet field strength (flux density) (±10%) | 5000 gauss |
| Flux density on extractor plate (±10%) | 4000 gauss |
| Depth | 127 mm |
| Width | 558 mm |
| Height | 520 mm |
| Weight | 50 kg |