|  | Cleanflow - automatisch discontinu reinigend - statisch - #200 mm Artikelnummer: E0164680 |
| --- | --- |

Kenmerken

| Omschrijving | Cleanflow - automatisch discontinu reinigend - statisch - #200 mm |
| --- | --- |
| Artikelnummer | E0164680 |
| Product key | SECA-2020-02V-ES-F1M-B-P-ST-B- |
| Suitable for food production | For non-perishable foods only |
| Max. particle size | 10 mm |
| Min. Fe-particle size that can be caught | 0,03 mm |
| Production stop required during ferrous particles disposal | Yes |
| Cleaning (ferrous particles disposal) method | Automatic |
| Ferrous particles disposal via | Pneumatically moving to and cleaning magnets in side channel |
| Ferrous particles disposal collection unit | No |
| Build-in length | 145 mm |
| Inlet shape | Square |
| Outlet shape | Square |
| Product inlet size | #200 mm |
| Product outlet size | #200 mm |
| Ferrous particles outlet size | #120 mm |
| Interface connection type/size | Square flanges with hole pattern acc. to drawing |
| Pneumatic connection | For 6 mm hose |
| Electrical connection | To solenoid valve |
| Electrical connection 2 | To proximity sensors for magnet (bar) detection (3x24 V) |
| Max. capacity | 30 m³/h |
| Deflector grid | No |
| Materials in contact with product | AISI304 (SS 1.4301) / AISI316L (SS 1.4404) |
| Material gasket inspection hatch(es) | Silicone VMQ, 60° Shore, EC1935/FDA, transparent |
| Material gasket top plate | Silicone VMQ, 60° Shore, EC1935/FDA, transparent |
| Material seal rings between housing and magnetic bars | Silicone 714177 coated, 70° Shore, EC1935/FDA, red |
| Top inspection hatch | Bolted cover plate AISI316L (SS 1.4404) |
| Side inspection hatch | Bolted cover plate PETG Griphen (transparent) |
| Surface finishing (interior/exterior) | S1: Grit blasted, Ra 3 µm |
| Extra wear-resistant coating | No |
| Welds inside the product channel | WM: Continuously welded |
| Welds outside the product channel | Wm: Continuously welded where possible |
| Max. operating temperature | 60 °C |
| Max. operating temperature in ATEX environment | 40 °C |
| Min./max. ambient temperature | -5 to 40 °C |
| Max. operating pressure (pipeline pressure) | 0,5 bar |
| ATEX marking | Ex II 1/2 D T130 °C |
| Suited for use in ATEX zone | 20, 21 and 22 inside, 21 and 22 outside |
| Magnetic system | Magnetic bar in tube |
| Magnet quality | Neodymium N-52 |
| Magnetic bars position / behaviour during production | Static, fixed position 1 row |
| Magnetic bar size | Ø52,4 mm (wall thickness 1,2 mm) |
| Number of magnetic bars | 2 |
| Flux density on magnet tube in contact with product (±10%) | 12000 gauss |
| Solenoid valve electric supply | 2x24 V DC |
| Pneumatic supply magnetic bars | 4 - 6 bar |
| Control | Electrical/pneumatic |
| Incl. Sigmatek PLC control | No |
| Incl. single bar in/out sensoring | Yes |
| Incl. all bars magnet in/out sensoring | No |
| Length | 1132 mm |
| Width | 611 mm |
| Height | 527 mm |
| Weight | 74 kg |