|  | Ovládací skříň - 110/230/400 V AC; 50/60 Hz Číslo výrobku: ESBP210002 |
| --- | --- |

Vlastnosti

| Popis | Ovládací skříň - 110/230/400 V AC; 50/60 Hz |
| --- | --- |
| Číslo výrobku | ESBP210002 |
| Produktový klíč | SPA-E-CB-NA-PLC-HW-PT1K0 |
| Control box type | Wall mounted |
| Suitable for | Electro bullet magnet - without valve box |
| Surface treatment/finishing | - |
| Colour | Grey (Light grey): RAL7035 |
| Min./max. ambient temperature | -10 to 30 °C |
| Dust/water protection class | IP54 |
| EMC directive 2014/30/EU compliant | EMC_2014-30EU |
| EN 60204-1 compliant | Yes |
| IEC 61439-1 compliant | Yes |
| IEC 61439-2 compliant | Yes |
| Supply voltage | 110/230/400 V AC + PE; 50/60 Hz |
| Supply current | 16 A |
| Maximum fuse | 16 A |
| Output power | 1 kW |
| Output voltage | 24 V DC |
| Output power to magnet | 1 kW |
| Output voltage to magnet | 24 V DC |
| Output current to magnet | 40 A |
| Including transformer | Yes |
| Including rectifier | Yes |
| Including Sigmatek controller with temp. & pos. monitoring | Yes |
| Including connection for PTC | Yes |
| Including demagnetizing relay | Yes |
| Including switch for local/remote control | Yes |
| Incl. pot. free input for magnet ON/OFF | Yes |
| Incl. pot. free output for: valve box in production position | Yes |
| Incl. pot. free output for: magnet on-off | Yes |
| Incl. pot. free output for: error message general | Yes |
| Incl. pot. free output for: error message temperature trip | Yes |
| Depth | 210 mm |
| Width | 600 mm |
| Height | 600 mm |
| Weight | 25 kg |