|  | Magswitch grijper Artikelnummer: MS-8140391 |
| --- | --- |

Kenmerken

| Omschrijving | Magswitch grijper |
| --- | --- |
| Artikelnummer | MS-8140391 |
| Product key | HMS-PLAY70x2 MDC-70MM |
| Type | PLAY |
| Switching | Pneumatic |
| Dimension lifting surface | 178x98 mm |
| Advised working load | 3150 N |
| Remark Advised working load | Under ideal conditions - with safety factor 3 acc. EN13155 |
| Max. magnetic tear off force | 9450 N |
| Remark max. magnetic tear off force | Under ideal conditions |
| Min. advised sheet thickness | 20 mm |
| Mounting interface | Threaded hole M8 (8x) |
| Pneumatic connection | Rc ¼" (2x) |
| Colour | Yellow / Black / - (not painted) |
| Min./max. ambient temperature | -20 to 80 °C |
| Operating pressure | 3 - 10 bar |
| Magnetic system | Permanent Neoflux® magnet |
| Magnetic layout (number of magnet poles) | 12 |
| Magnet quality | Neodymium (NdFeB) |
| Incl. replaceable pole shoes (for alt. product shapes) | Yes |
| Length | 194 mm |
| Width | 95 mm |
| Height | 282 mm |
| Weight | 13 kg |