|  | Magswitch grijper PLAY70x3 Artikelnummer: MS-8140544 |
| --- | --- |

Kenmerken

| Omschrijving | Magswitch grijper PLAY70x3 |
| --- | --- |
| Artikelnummer | MS-8140544 |
| Product key | HMS-PLAY70x3 MDC-70MM |
| Type | PLAY |
| Switching | Pneumatic |
| Dimension lifting surface | 255x98 mm |
| Advised working load | 4900 N |
| Remark Advised working load | Under ideal conditions - with safety factor 3 acc. EN13155 |
| Max. magnetic tear off force | 14720 N |
| Remark max. magnetic tear off force | Under ideal conditions |
| Min. advised sheet thickness | 20 mm |
| Mounting interface | Threaded hole M8 (16x) |
| Pneumatic connection | Rc ¼" (2x) |
| Colour | Yellow / Black / - (not painted) |
| Min./max. ambient temperature | -20 to 80 °C |
| Operating pressure | 3 - 10 bar |
| Magnetic system | Permanent Neoflux™ magnet |
| Magnetic layout (number of magnet poles) | 12 |
| Magnet quality | Neodymium (NdFeB) |
| Incl. replaceable pole shoes (for alt. product shapes) | Yes |
| Length | 286 mm |
| Width | 125 mm |
| Height | 301 mm |
| Weight | 22 kg |