|  | Ferriet bovenbandmagneet - 1000 mm Artikelnummer: ROFC100330 |
| --- | --- |

Kenmerken

| Omschrijving | Ferriet bovenbandmagneet - 1000 mm |
| --- | --- |
| Artikelnummer | ROFC100330 |
| Product key | ROP-F3-C-100-W-G-L-B-B-B-NA |
| Suitable for cross mounting | Yes |
| Suitable for inline mounting | No |
| Min. Fe-particle size that can be caught | 1 mm |
| Advised max. installation height | 320 mm |
| Ferrous particles disposal method | Automatic - continuous |
| Suitable for belt width | 1000 mm |
| Installation/transport interface | Lifting eye Ø25 mm (4x) |
| Material framework | Steel |
| Material covering | Stainless steel |
| Surface treatment/finishing magnet(s) | Painted |
| Surface treatment/finishing rollers | Painted |
| Surface treatment/finishing frame | Painted |
| Surface treatment/finishing covers | Grit blasted |
| Colour magnet(s) | Red (Traffic red): RAL3020 |
| Colour motor | Black (Jet Black): RAL9005 |
| Colour frame | Dark grey (Black grey): RAL7021 |
| Colour covering | - (Stainless steel) |
| Colour rollers | Dark grey (Black grey): RAL7021 |
| Max. operating temperature | 100 °C |
| Min./max. ambient temperature | -20 to 40 °C |
| Conveying length | 1730 mm |
| Belt width | 650 mm |
| Belt speed | 1,7 m/s |
| Belt type | Rubber, with cleats (EP400/3 4+2) |
| Belt cleat type | T30/10 |
| No. of cleats | 6 |
| Magnetic system | Permanent ferrite magnet |
| Magnet quality | Anisotropic ferrite F-33 |
| Magnet dimensions | 1250x620x256 mm |
| Main pole dimensions | 1040x620x256 mm |
| Motor drive | SEW electro motor with gear box |
| Motor power | 2,2 kW |
| Motor frequency | 50 Hz |
| Motor voltage | 230 (Δ) / 400 (Y) V AC |
| Motor position / operating side | Left (seen in transport direction) |
| Motor rotational speed (out) | 102 rpm |
| Motor dust/water protection class | IP55 |
| Duty cycle | 100%, 24 hours/day |
| Including central greasing point on both sides for bearings | No |
| Length | 2215 mm |
| Width | 1100 mm |
| Height | 430 mm |
| Weight | 1032 kg |