|  | Neodymium bovenbandmagneet - 800 mm Artikelnummer: RONC080551 |
| --- | --- |

Kenmerken

| Omschrijving | Neodymium bovenbandmagneet - 800 mm |
| --- | --- |
| Artikelnummer | RONC080551 |
| Product key | ROP-N5-C-080-W-H-R-B-B-M-NA |
| Suitable for cross mounting | Yes |
| Suitable for inline mounting | Yes |
| Min. Fe-particle size that can be caught | 1 mm |
| Advised max. installation height | 420 mm |
| Ferrous particles disposal method | Automatic - continuous |
| Suitable for belt width | 800 mm |
| Installation/transport interface | Lifting eye Ø22 mm (8x) |
| Material framework | Stainless steel |
| Material covering | Stainless steel |
| Surface treatment/finishing magnet(s) | Painted |
| Surface treatment/finishing rollers | Painted |
| Surface treatment/finishing frame | Grit blasted |
| Surface treatment/finishing covers | Grit blasted |
| Colour rollers | Dark grey (Black grey): RAL7021 |
| Max. operating temperature | 60 °C |
| Min./max. ambient temperature | -20 to 40 °C |
| Conveying length | 1320 mm |
| Belt width | 1000 mm |
| Belt speed | 2 m/s |
| Belt type | Rubber, with cleats (EP250/2 3+1,5) |
| Belt cleat type | T50/20 |
| No. of cleats | 10 |
| Magnetic system | Permanent Neoflux™ magnet |
| Magnet quality | Neodymium N-38 |
| Magnet dimensions | 1000x800x120 mm |
| Main pole dimensions | 800x800x120 mm |
| Motor drive | Danfoss hydromotor |
| Motor position / operating side | Right (seen in transport direction) |
| Duty cycle | 100%, 24 hours/day |
| Including central greasing point on both sides for bearings | No |
| Length | 1700 mm |
| Width | 1430 mm |
| Height | 320 mm |
| Weight | 810 kg |