|  | Elektro bovenbandmagneet - 1000 mm Artikelnummer: SEEB100023 |
| --- | --- |

Kenmerken

| Omschrijving | Elektro bovenbandmagneet - 1000 mm |
| --- | --- |
| Artikelnummer | SEEB100023 |
| Product key | ROE-05-C-100-W-G-R-B-R-B-B-NA |
| Suitable for cross mounting | Yes |
| Suitable for inline mounting | Yes |
| Min. Fe-particle size that can be caught | 3 mm |
| Advised max. installation height | 360 mm |
| Ferrous particles disposal method | Automatic - continuous |
| Installation/transport interface | Lifting eye Ø35 mm (12x) |
| Material framework | Steel |
| Material covering | Stainless steel |
| Surface treatment/finishing magnet(s) | Painted |
| Surface treatment/finishing rollers | Painted |
| Surface treatment/finishing frame | Painted |
| Surface treatment/finishing covers | Grit blasted |
| Colour magnet(s) | Red (Traffic red): RAL3020 |
| Colour motor | Black (Jet Black): RAL9005 |
| Colour frame | Dark grey (Black grey): RAL7021 |
| Colour covering | - (Stainless steel) |
| Max. operating temperature | 80 °C |
| Min./max. ambient temperature | -20 to 40 °C |
| Conveying length | 2250 mm |
| Belt width | 1000 mm |
| Belt speed | 1,7 m/s |
| Belt type | Rubber, with cleats (EP400/3 4+2) |
| Belt cleat type | T100/10 |
| No. of cleats | 12 |
| Magnetic system | Electromagnet with unforced oil cooling |
| Magnet dimensions | 1555x950x430 mm |
| Main pole dimensions | 1000x950x430 mm |
| Magnet power cold | 5800 W |
| Magnet power warm | 4300 W |
| Magnet dust/water protection class | IP67 |
| Motor drive | SEW electro motor with gear box |
| Motor power | 2,2 kW |
| Motor frequency | 50 Hz |
| Motor voltage | 230 (Δ) / 400 (Y) V AC |
| Motor position / operating side | Right (seen in transport direction) |
| Motor rotational speed (out) | 69 rpm |
| Motor dust/water protection class | IP55 |
| Duty cycle | 100%, 24 hours/day |
| Associated control unit (not included) | ESBB055000 |
| Including central greasing point on both sides for bearings | No |
| Rotation detection belt | XSAV11373EX 2D M30 |
| Length | 2912 mm |
| Width | 1623 mm |
| Height | 896 mm |
| Weight | 2900 kg |