|  | Pneumatische magneetgrijper - 160 mm Artikelnummer: TPGC160078 |
| --- | --- |

Kenmerken

| Omschrijving | Pneumatische magneetgrijper - 160 mm |
| --- | --- |
| Artikelnummer | TPGC160078 |
| Product key | HGR-SQ-160-FL-F-G-S |
| Switching | Pneumatic |
| Dimension lifting surface | Ø160 mm |
| Advised working load | 1170 N |
| Remark Advised working load | Under ideal conditions - with safety factor 3 acc. EN13155 |
| Max. magnetic tear off force | 3500 N |
| Remark max. magnetic tear off force | Under ideal conditions |
| Min. advised sheet thickness | 3 mm |
| Mounting interface | Threaded hole M8 (8x) + Threaded hole M12 |
| Pneumatic connection | G¼" (2x) |
| Material housing | Aluminium / AISI316L (SS 1.4404) (screw cap) |
| Surface treatment/finishing | Anodized |
| Colour | Red |
| Min./max. ambient temperature | 0 to 70 °C |
| Operating pressure | Max. 4 bar |
| Air consumption per cycle (1x up, 1x down) | 5,208 |
| Magnetic system | Permanent Neoflux® magnet |
| Magnetic layout (number of magnet poles) | 12 |
| Magnet quality | Neodymium N-45 |
| Incl. friction pad on magnetic surface | No |
| Length | 170 mm |
| Width | 170 mm |
| Height | 103 mm |
| Weight | 5,7 kg |