|  | Separator z magnesami zewnetrznymi – czyszczenie automatyczne - 240 m³/h Numer artykułu: E0102164 |
| --- | --- |

Właściwości

| Opis | Separator z magnesami zewnetrznymi – czyszczenie automatyczne - 240 m³/h |
| --- | --- |
| Numer artykułu | E0102164 |
| Klucz produktu | SBPA-4330-N-F1E-B-D3 |
| Max. particle size | 50 mm |
| Min. Fe-particle size that can be caught | 1 mm |
| Production stop required during ferrous particles disposal | Yes |
| Cleaning (ferrous particles disposal) method | Automatic |
| Ferrous particles disposal via | Pneumatically moving out magnets + extractor plates |
| Build-in length | 600 mm |
| In-/outlet shape | Rectangular |
| Product inlet size | 426x300 mm |
| Product outlet size | 426x300 mm |
| Interface connection type/size | Rectangular flanges with hole pattern acc. to drawing |
| Max. capacity | 240 m³/h |
| Deflector | Removable |
| Materials in contact with product | AISI316L (SS 1.4404) |
| Material seal between housing and door/magnet unit | Silicone VMQ, 20° Shore, EC1935/FDA, white |
| Surface finishing (interior/exterior) | S1: Grit blasted, Ra 3 µm |
| Extra wear-resistant coating | No |
| Welds inside the product channel | WB: Welded shut |
| Welds outside the product channel | WB: Welded shut |
| Max. operating temperature | 80 °C |
| Min./max. ambient temperature | -20 to 60 °C |
| Magnetic system | Permanent Neoflux™ magnet |
| Magnet quality | Neodymium N-35 |
| Magnet dimensions | 380x204x55 mm |
| Number of magnets | 2 |
| Magnet field strength (flux density) (±10%) | 5000 gauss |
| Flux density on extractor plate (±10%) | 4000 gauss |
| Depth | 773 mm |
| Width | 1115 mm |
| Height | 676 mm |
| Weight | 205 kg |