|  | EddyFines - Wirowo-pradowy separator - 1500 mm Numer artykułu: E0139716 |
| --- | --- |

Właściwości

| Opis | EddyFines - Wirowo-pradowy separator - 1500 mm |
| --- | --- |
| Numer artykułu | E0139716 |
| Klucz produktu | REC-15-E38HI-F-C20-FMS-PU-R |
| Incl. magnetic drum separator | Yes |
| Incl. magnetic head pulley separator | No |
| Incl. magnetic overband separator | No |
| Feeder module | None |
| Max. particle size | 100 mm |
| Separates non-magnetic particles | 0-10+ mm |
| Length head pulley separator belt | 2015 mm |
| Separation box | Medium |
| Separation flap type separation box | Fines, Manganese steel |
| Separation box flap operation | Manual by lever |
| Separation flap type head pulley separator | Stainless steel |
| Working width | 1500 mm |
| Rotational speed eddy current magnet roller | 3000 rpm |
| Material framework | Steel |
| Material covering | Steel |
| Material slide plates | Stainless steel |
| Surface treatment/finishing | S6: Powder coated |
| Colour P1: frame + covering | Dark grey (Black grey): RAL7021 |
| Colour P2: side panels | Red (Traffic red): RAL3020 |
| Max. operating temperature | 60 °C |
| Min./max. ambient temperature | -20 to 40 °C |
| Eddy current belt height | 1032 mm |
| Head pulley separator belt speed | 0 - 2,0 m/s |
| Eddy current belt speed | 0 - 2,5 m/s |
| Belt type eddy current separator | 2,4 mm Polyurethane, white, 94° Shore |
| Belt type head pulley separator | 6,5 mm rubber, black (EP250/2 3+1,5) |
| Belt scraper eddy current separator | Rubber (NBR) |
| Belt scraper head pulley separator | Steel |
| Eddy current magnet roller type | 38 poles High Intensity, eccentric Ø500/Ø700 magnet rotor |
| Magnet quality | Neodymium (NdFeB) |
| Flux density eddy current magnet roller, on belt (±10%) | 3500 gauss |
| Flux density magnetic head pulley (±10%) | 1800 gauss |
| Control unit power supply | 3x400 V AC + PE; 50 Hz |
| Motor drive eddy current belt | Van der Graaf drum motor |
| Motor drive eddy current magnet roller | AEG electro motor |
| Motor drive head pulley separator | SEW electro motor with gear box |
| Motor power eddy current belt | 2,2 kW |
| Motor current eddy current belt | 4,7 A |
| Motor frequency eddy current belt | 50 Hz |
| Motor voltage eddy current belt | 230 (Δ) / 400 (Y) V AC |
| Motor power eddy current magnet roller | 11 kW |
| Motor frequency eddy current magnet roller | 50 Hz |
| Motor voltage eddy current magnet roller | 400 V AC (Δ) / 690 V AC (Y) V AC |
| Motor power head pulley separator | 3 kW |
| Motor frequency head pulley separator | 50 Hz |
| Motor voltage head pulley separator | 230 (Δ) / 400 (Y) V AC |
| Motor position / operating side | Right (seen in transport direction) |
| Belt motor dust/water protection class | IP66 |
| Head pulley separator motor dust/water protection class | IP55 |
| Eddy current magnet roller motor dust/water protection class | IP55 |
| Associated control unit (not included) | E0131085/E0131086/E0131087/E0131088 |
| Including central greasing point on both sides for bearings | No |
| Electrical connector plate | Yes (central plate with electrical connection plugs) |
| Thermal protection eddy current roller | Yes |
| Rotation detection eddy current belt | IFM II504A-EXII2D M30 |
| Rotation detection head pulley separator | IFM IIK3010-EXII2D M30 |
| Vibration sensor eddy current magnet roller | S/N 490250 |
| Length | 5916 mm |
| Width | 2808 mm |
| Height | 1994 mm |
| Weight | 4946 kg |