|  | Skrzynka sterownicza - 3x400 V AC + PE; 50/60 Hz; 15 kW Numer artykułu: ESBB104000 |
| --- | --- |

Właściwości

| Opis | Skrzynka sterownicza - 3x400 V AC + PE; 50/60 Hz; 15 kW |
| --- | --- |
| Numer artykułu | ESBB104000 |
| Klucz produktu | ROE-E-CB-NA-LC-HW-PT10K4 |
| Control box type | Floor mounted |
| Suitable for | Electro overband magnet |
| Surface treatment/finishing | - |
| Colour | Grey (Light grey): RAL7035 |
| Min./max. ambient temperature | -10 to 30 °C |
| Dust/water protection class | IP54 |
| EMC directive 2014/30/EU compliant | EMC_2014-30EU |
| EN 60204-1 compliant | Yes |
| IEC 61439-1 compliant | Yes |
| IEC 61439-2 compliant | Yes |
| Supply voltage | 3x400 V AC + PE; 50/60 Hz |
| Supply current | 22 A |
| Maximum fuse | 25 A |
| Output power | 13 kW |
| Output power to magnet | 10,4 kW |
| Output voltage to magnet | 232...255 V DC |
| Output current to magnet | 49 A |
| Output power to belt drive | 3,0 - 4,0 kW |
| Output voltage to belt drive | 400 V AC |
| Output current to belt drive | 5,5 - 8 A |
| Including rectifier | Yes |
| Including switch for local/remote control | Yes |
| Including safety switch | Yes, for transformer and motor |
| Main switch | Black/grey |
| Indicator light 'Running' (green) | Yes |
| Indicator light 'Alarm' (red) | Yes |
| Indicator light 'Standby' (white) | Yes |
| Emergency stop | Yes |
| Push button 'Reset emergency stop' | Yes |
| Push button ON | Yes |
| Push button OFF | Yes |
| Switch for local/remote control | Yes |
| Thermal protection | Yes |
| Depth | 400 mm |
| Width | 800 mm |
| Height | 1200 mm |
| Weight | 180 kg |