|  | Skrzynka sterownicza - 110/230/400 V AC; 50/60 Hz Artikel Nummer: ESBP310003 |
| --- | --- |

Eigenschaften

| Beschreibung | Skrzynka sterownicza - 110/230/400 V AC; 50/60 Hz |
| --- | --- |
| Artikel Nummer | ESBP310003 |
| Produktschlüssel | SPA-E-CB-X2-PLC-HW-PT1K0 |
| Control box type | Wall mounted |
| Suitable for | Electro bullet magnet ATEX - with valve box |
| Material enclosure and door | Stainless steel |
| Surface treatment/finishing | - |
| Colour | Grey (Light grey): RAL7035 |
| Min./max. ambient temperature | 0 to 40 °C |
| ATEX marking | Ex II 2D tb IIIC T130°C |
| Suited for use in ATEX zone | 21 and 22 |
| Dust/water protection class | IP54 |
| EMC directive 2014/30/EU compliant | EMC_2014-30EU |
| EN 60079-0 compliant | Yes |
| EN 60079-31 compliant | Yes |
| EN 60204-1 compliant | Yes |
| IEC 61439-1 compliant | Yes |
| IEC 61439-2 compliant | Yes |
| Supply voltage | 110/230/400 V AC + PE; 50/60 Hz |
| Supply current | 16 A |
| Maximum fuse | 16 A |
| Output power | 1 kW |
| Output voltage | 48 V DC |
| Output power to magnet | 1 kW |
| Output voltage to magnet | 48 V DC |
| Output current to magnet | 20 A |
| Including transformer | Yes |
| Including rectifier | Yes |
| Including Sigmatek controller with temp. & pos. monitoring | Yes |
| Including connection for PTC | Yes |
| Including demagnetizing relay | Yes |
| Including switch for local/remote control | Yes |
| Incl. pot. free input for magnet ON/OFF | Yes |
| Incl. pot. free output for: magnet on/off/error/valve closed | Yes |
| Depth | 210 mm |
| Width | 600 mm |
| Height | 600 mm |
| Weight | 25 kg |