|  | Separator z magnesami zewnetrznymi — reczne szybkie czyszczenie - 40 m³/h Numer artykułu: SBPN151500 |
| --- | --- |

Właściwości

| Opis | Separator z magnesami zewnetrznymi — reczne szybkie czyszczenie - 40 m³/h |
| --- | --- |
| Numer artykułu | SBPN151500 |
| Klucz produktu | SBP-1515-N-F1E-B-B |
| Max. particle size | 50 mm |
| Min. Fe-particle size that can be caught | 0,5 mm |
| Production stop required during ferrous particles disposal | Yes |
| Cleaning (ferrous particles disposal) method | Manual |
| Ferrous particles disposal via | Manually by moving away the extractor plate |
| Build-in length | 400 mm |
| In-/outlet shape | Square |
| Product inlet size | #150 mm |
| Product outlet size | #150 mm |
| Interface connection type/size | Square flanges with hole pattern acc. to drawing |
| Max. capacity | 40 m³/h |
| Deflector | Removable |
| Materials in contact with product | AISI304 (SS 1.4301) |
| Material seal between housing and door/magnet unit | Silicone VMQ, 40° Shore, EC1935/FDA, red |
| Surface finishing (interior/exterior) | S1: Grit blasted, Ra 3 µm |
| Surface treatment/finishing magnet(s) | Painted |
| Extra wear-resistant coating | No |
| Welds inside the product channel | WE: Continuously welded |
| Welds outside the product channel | WE: Intermittently welded |
| Colour magnet(s) | Red (Traffic red): RAL3020 |
| Colour housing | - (not painted) |
| Max. operating temperature | 80 °C |
| Min./max. ambient temperature | -20 to 60 °C |
| Magnetic system | Permanent Neoflux™ magnet |
| Magnet quality | Neodymium N-38 |
| Magnet dimensions | 271x307x48 mm |
| Number of magnets | 2 |
| Flux density on extractor plate (±10%) | 4800 gauss |
| Flux density on extractor wedge (±10%) | 10000 gauss |
| Safety device between housing and door/magnet unit | No |
| Depth | 377 mm |
| Width | 475 mm |
| Height | 417 mm |
| Weight | 65 kg |