|  | Separator przeplywowy – czyszczenie reczne – statyczny - Ø250 mm Numer artykułu: SECF250638 |
| --- | --- |

Właściwości

| Opis | Separator przeplywowy – czyszczenie reczne – statyczny - Ø250 mm |
| --- | --- |
| Numer artykułu | SECF250638 |
| Klucz produktu | SECF-B-D250-11E-N-F1E-W-B-B-SR |
| Suitable for food production | For non-perishable foods only |
| Max. particle size | 10 mm |
| Min. Fe-particle size that can be caught | 0,03 mm |
| Production stop required during ferrous particles disposal | Yes |
| Cleaning (ferrous particles disposal) method | Manual |
| Ferrous particles disposal via | Manually taking out magnets + pulling them out of extractor |
| Build-in length | 250 mm |
| Inlet shape | Round |
| Outlet shape | Round |
| Product inlet size | Ø250 mm |
| Product outlet size | Ø250 mm |
| Interface connection type/size | Hole pattern according to: DN250 acc. EN 1092-1 PN10 |
| Max. capacity | 30 m³/h |
| Materials in contact with product | AISI316 (SS 1.4401) / AISI316L (SS 1.4404) |
| Material seal between housing and door/magnet unit | Silicone VMQ, 40° Shore, EC1935/FDA, red |
| Surface finishing (interior/exterior) | S1: Grit blasted, Ra 3 µm |
| Extra wear-resistant coating | No |
| Welds inside the product channel | WE: Continuously welded |
| Welds outside the product channel | WE: Intermittently welded |
| Max. operating temperature | 60 °C |
| Min./max. ambient temperature | -20 to 60 °C |
| ATEX marking | Free of own ignition sources |
| Suited for use in ATEX zone | 20, 21 and 22 |
| Magnetic system | Magnetic bar in extractor tube |
| Magnet quality | Neodymium N-42 |
| Magnetic bars position / behaviour during production | Static, fixed positon in 2 rows, placed in cascade |
| Magnetic bar size | Extractor tube Ø25 mm; magnetic bar Ø23 mm |
| Number of magnetic bars | 11 (5+6) |
| Flux density on (internal) magnetic bar (±10%) | 10700 gauss |
| Flux density on magnet tube in contact with product (±10%) | 8000 gauss |
| Safety device between housing and door/magnet unit | No |
| Length | 395 mm |
| Width | 455 mm |
| Height | 250 mm |
| Weight | 36 kg |