|  | Magnes rurowy trwaly - ferryt - 380 m³/h Numer artykułu: SPPE000066 |
| --- | --- |

Właściwości

| Opis | Magnes rurowy trwaly - ferryt - 380 m³/h |
| --- | --- |
| Numer artykułu | SPPE000066 |
| Klucz produktu | SPP-6-FI-FO-ST-B-BB |
| Tapered in-/outlet | Yes |
| Min. Fe-particle size that can be caught | 0,05 mm |
| Production stop required during ferrous particles disposal | Yes |
| Cleaning (ferrous particles disposal) method | Manual |
| Ferrous particles disposal via | Manually by wiping |
| Build-in length | 1200 mm |
| Inlet shape | Round |
| Outlet shape | Round |
| Product inlet size | Ø500 mm |
| Product outlet size | Ø500 mm |
| Inlet interface connection type/size | DN500 acc. DIN 24154 R4 |
| Outlet interface connection type/size | DN500 acc. DIN 24154 R4 |
| Max. capacity | 380 m³/h |
| Adjustable fasteners on cleaning/inspection door | Yes |
| Materials in contact with product | Steel / AISI304 (SS 1.4301) / Nickel plated steel |
| Material seal between housing and door/magnet unit | Silicone sponge rubber VMQ, white |
| Surface treatment/finishing | Painted |
| Extra wear-resistant coating | No |
| Welds inside the product channel | WB: Welded shut |
| Welds outside the product channel | WB: Welded shut |
| Colour housing | Grey (Platinum grey): RAL7036 |
| Colour door | Dark grey (Umbra grey): RAL7022 |
| Max. operating temperature | 100 °C |
| Max. operating temperature in ATEX environment | 80 °C |
| Min./max. ambient temperature | -20 to 40 °C |
| ATEX marking | Ex II 1/2 D T135 °C |
| Suited for use in ATEX zone | 20, 21 and 22 inside, 21 and 22 outside |
| Magnetic system | Permanent ferrite magnet |
| Magnet quality | Anisotropic ferrite F-33 |
| Magnet diameter | 500 mm |
| Angle magnet cone | 70 ° |
| Magnet field strength (flux density) (±10%) | 7000 gauss |
| Safety device between housing and door/magnet unit | No |
| Depth | 925 mm |
| Width | 1075 mm |
| Height | 1200 mm |
| Weight | 500 kg |