|  | Magnetyczny separator bebnowy w obudowie — typ podstawowy - 120 m³/h Numer artykułu: SRTK080044 |
| --- | --- |

Właściwości

| Opis | Magnetyczny separator bebnowy w obudowie — typ podstawowy - 120 m³/h |
| --- | --- |
| Numer artykułu | SRTK080044 |
| Klucz produktu | SETK-084-SD-B-30S2-NA-FZB-B-B |
| Max. particle size | 75 mm |
| Min. Fe-particle size that can be caught | 0,5 mm |
| Production stop required during ferrous particles disposal | No |
| Cleaning (ferrous particles disposal) method | Automatic |
| Ferrous particles disposal via | Discharge chute |
| In-/outlet shape | Rectangular |
| Product inlet size | 800x200 mm |
| Product outlet size | 930x250 mm |
| Dust extraction opening size | 800x60 mm |
| Ferrous particles outlet size | 930x150 mm |
| Max. capacity | 120 m³/h |
| Rotational speed | 21 rpm |
| Surface speed | 25,5 m/min |
| Shell thickness | 3 mm |
| Materials in contact with product | Steel / rubber (NBR) |
| Surface treatment/finishing | Painted |
| Welds inside the product channel | WB: Welded shut |
| Welds outside the product channel | WB: Welded shut |
| Colour housing | White (Traffic white): RAL 9016 |
| Colour side panels / inspection hatch | Blue (Ultramarine blue): RAL 5002 |
| Max. operating temperature | 60 °C |
| Min./max. ambient temperature | -20 to 40 °C |
| Magnet width | 800 mm |
| No. of cleats | 2 |
| Magnetic system | Permanent ferrite magnet with flux boost |
| Magnet quality | Anisotropic ferrite F-33 + Neodymium N-38 |
| Magnet diameter | 406 mm |
| Flux density on shell (±10%) | 3000 gauss |
| Motor drive | SEW electro motor with gear box |
| Motor power | 0,55 kW |
| Motor frequency | 50 Hz |
| Motor voltage | 230 (Δ) / 400 (Y) V AC |
| Motor dust/water protection class | IP54 |
| Depth | 730 mm |
| Width | 1460 mm |
| Height | 650 mm |
| Weight | 400 kg |