|  | Magnetyczny separator bebnowy w obudowie — typ podstawowy - 250 m³/h Numer artykułu: SRTK140054 |
| --- | --- |

Właściwości

| Opis | Magnetyczny separator bebnowy w obudowie — typ podstawowy - 250 m³/h |
| --- | --- |
| Numer artykułu | SRTK140054 |
| Klucz produktu | SETK-145-SD-B-30S2-NA-FZB-B-B |
| Max. particle size | 100 mm |
| Min. Fe-particle size that can be caught | 0,5 mm |
| Production stop required during ferrous particles disposal | No |
| Cleaning (ferrous particles disposal) method | Automatic |
| In-/outlet shape | Rectangular |
| Product inlet size | 1400x400 mm |
| Product outlet size | 1540x500 mm |
| Dust extraction opening size | - mm |
| Ferrous particles outlet size | 1540x390 mm |
| Max. capacity | 350 m³/h |
| Rotational speed | 20 rpm |
| Surface speed | 38 m/min |
| Shell thickness | 3 mm |
| Materials in contact with product | Stainless steel |
| Surface treatment/finishing | Painted |
| Welds inside the product channel | WB: Welded shut |
| Welds outside the product channel | WB: Welded shut |
| Colour housing | White (Traffic white): RAL 9016 |
| Colour side panels / inspection hatch | Blue (Ultramarine blue): RAL 5002 |
| Max. operating temperature | 60 °C |
| Min./max. ambient temperature | -20 to 40 °C |
| Magnet width | 1400 mm |
| Magnetic system | Permanent ferrite magnet |
| Magnet quality | Anisotropic ferrite GSFD-33, Br 4100 Gauss (at 20°C), Tmax |
| Magnet diameter | 610 mm |
| Motor drive | Suitable for Electro motor with gear box: SEW FF77DRN90L4 |
| Motor power | 1,5 kW |
| Motor frequency | 50 Hz |
| Motor voltage | 230 (Δ) / 400 (Y) V AC |
| Motor dust/water protection class | IP54 |
| Depth | 2260 mm |
| Width | 1200 mm |
| Height | 900 mm |
| Weight | 1225 kg |