|  | Pneumatyczny chwytak magnetyczny - 100 mm Numer artykułu: TPGC100378 |
| --- | --- |

Właściwości

| Opis | Pneumatyczny chwytak magnetyczny - 100 mm |
| --- | --- |
| Numer artykułu | TPGC100378 |
| Klucz produktu | HGR-SQ-100-FL-RH-G-F |
| Switching | Pneumatic |
| Dimension lifting surface | Ø100 mm |
| Advised working load | 400 N |
| Remark Advised working load | Under ideal conditions - with safety factor 3 acc. EN13155 |
| Max. magnetic tear off force | 1200 N |
| Remark max. magnetic tear off force | Under ideal conditions |
| Min. advised sheet thickness | 3 mm |
| Mounting interface | Threaded hole M6 (8x) + Threaded hole M10 |
| Pneumatic connection | G1/8" (2x) |
| Material housing | Aluminium / AISI316L (SS 1.4404) (screw cap) |
| Surface treatment/finishing | Nickel plated |
| Colour | Grey |
| Min./max. ambient temperature | 0 to 120 °C |
| Operating pressure | Max. 4 bar |
| Air consumption per cycle (1x up, 1x down) | 1,352 |
| Magnetic system | Permanent Neoflux® magnet |
| Magnetic layout (number of magnet poles) | 8 |
| Magnet quality | Neodymium N-38EH |
| Incl. friction pad on magnetic surface | No |
| Length | 100 mm |
| Width | 100 mm |
| Height | 70 mm |
| Weight | 1,5 kg |